Water Purification

- Large-Scale Purification:

  - Process:

    - Conducted by water treatment plants.

    - Involves filtration, disinfection, and chemical treatment.

  - Objective:

    - Provide clean water to a large population.

    - Ensure water meets quality standards for consumption.

  - Advantages:

    - Treats water from natural sources like rivers or reservoirs.

    - Ensures water is safe for consumption by homes, schools, and businesses.

- Small-Scale Purification:

  - Methods:

    - Boiling, chemical disinfection, and filtration.

  - Purpose:

    - Treat water at a smaller level, often for individual households or communities.

    - Remove contaminants and ensure safe drinking water.

  - Advantages:

    - Suitable for areas with limited access to large-scale purification facilities.

    - Simple and effective methods for household use.

Household Purification Methods:

- Boiling:

  - Bring water to a rolling boil for one minute to kill harmful bacteria and pathogens.

  - Simple and reliable method for ensuring safe drinking water.

- Chemical Disinfection:

  - Use substances like bleaching powder, chlorine solution, or chlorine tablets.

  - Follow recommended amounts and contact time for effective disinfection.

  - Kills pathogens and makes water safe for consumption.

- Filtration:

  - Use water filters with fine mesh or activated carbon to physically remove impurities.

  - Choose reliable filters suitable for bacterial removal and ensure proper maintenance.

Double Pot Method:

- Description:

  - Involves using two cylindrical pots, one placed inside the other.

  - Outer pot: Height of 30 cm, diameter of 25 cm, with a hole positioned 4 cm above the bottom.

  - Inner pot filled with a mixture of bleaching powder and coarse sand.

  - Placed inside the outer pot, sealed with polythene foil.

- Procedure:

  - Lowered into a well using a rope, ensuring immersion at least 1 meter below the water level.

  - Effective for about 2-3 weeks in a well containing approximately 4500 liters of water.

  - Provides a simple and practical method for water purification in communities.

Water Purification on a Large Scale:

- Storage:

  - Water drawn from sources and stored in reservoirs.

  - Acts as a reserve, protecting water from pollution.

  - Natural purification occurs during storage:

    - Physical: Suspended impurities settle due to gravity.

    - Chemical: Changes like ammonia decrease, nitrates increase.

  - Bacterial count decreases over time, enhancing safety.

  - Proper maintenance crucial to prevent algae growth.

- Filtration:

  - Slow Sand Filters:

    - Consist of supernatant water, sand bed, and biological layer.

    - Allows partial purification through sedimentation, adsorption, and bacterial action.

    - Simple construction, lower cost, and high-quality filtered water.

  - Rapid Sand Filters:

    - Types: gravity and pressure filters.

    - Process involves chemical coagulation, flocculation, sedimentation, and rapid sand filtration.

    - Requires backwashing to maintain efficiency:

      - Reverses water flow to dislodge accumulated impurities.

      - Frequent washing needed, typically daily or weekly.

    - Advantages: Handles raw water directly, higher filtration rate, occupies less space, and offers flexibility in operation.

- Chlorination:

  - Works alongside filtration to kill pathogenic bacteria.

  - Forms hydrochloric and hydrochlorous acids in water.

  - Hypochlorous acid primarily responsible for disinfection.

  - Methods:

    - Chlorine gas: Affordable, efficient, requires specialized equipment.

    - Chloramines: Less chlorinous taste, slower action, less commonly used.

    - Perchloron or HTH: Calcium compound, high chlorine content, used for disinfection.

  - Break point chlorination: Chlorine dose increase leading to residual chlorine increase until a saturation point is reached.

Large-scale water purification involves storage, filtration, and chlorination, ensuring water safety and quality for consumption on a broad scale. Proper implementation and maintenance of these methods are essential for providing clean water to communities.

Top of Form

Top of Form